BF-V5功能操作说明                       V5 Function Operation Instruction

一、Function Introduction

◆Frequency

UHF：400–470 MHZ (RX/TX)

◆ Up to 16 Channels

◆ Chinese/English Language

◆ 50 Groups CTCSS，105 Groups of standard DCS

◆ Time-out timer(TOT)

◆ Broad band/Narrow band

◆ Battery saving function

◆ Busy Channel Lockout (BCL)

◆ Low Battery Voltage Warning

◆ Programmable by PC

二、Function Operation

静噪级别  SQUELCH LEVEL

The purpose of Squelch is to mute the speaker when no signals are present (Squelch OFF). With the squelch level correctly set, you will hear sound only while actually receiving signals (Squelch ON). The squelch level can be adjusted via program software. The frequency of squelch is 0 (on) -9 (deep), steps: 1.

扫描开关 SCANNING SWITCH

- When walkie-talkie is set as scan via software, turn the channel selector to channel 16,after the 5 sec, the radio will automatically detect the activity of scanning channel from CH1 to CH15 channel (each channel can bet set scan or not via software) , When the signal is scanned channel, the radio will automatically stop on the channel for the call.

- a.	Walkie-talkie will stay on the signal channel, when the signal disappears and then after 5 sec, continue scanning the next channel.

- b.	If there are less than 2 channels, it cannot scan.

- c.	On channel 16, press and hold the PTT and side key, it will turn on the SCAN or turn off the SCAN.

- d.	If the FM radio function is activated, it cannot scan.

声控发射   VOX

- a. VOX function is effective when the function is turn on.

- b. VOX gain level: When the sound level is higher than the set level of VOX, VOX automatically transmit. The VOX gain level can adjust from 1 to 9 levels.

- c. When a signal is received, VOX will not transmit even if the sound level is higher than the set VOX gain level.

- d. On channels CH1 - CH5, press and hold the PTT and side key, then switch on the radio, thus to activate the VOX function ON/OFF.

Battery Saving

a.ON: Entering into the battery saving state automatically without any operation for 10 seconds

b.OFF: Battery saving is off.

Busy Channel Lockout (BCL)

Switch on or off each effective channel busy lockout function via PC programming software. When open this setting, signal from the current channel is banned to transmit.

Prompt Tone

a. The beep is on, and the "Di" sounds is emitted for every rotation of the channel

b. The beep is off, and no "Di" sounds for every rotation of the channel

高/低电压警示  High/low voltage

a．电压过高：在高电禁止发射时，电压过高，按PTT键或声控发射时，提示“嘀”音。

b．低电报警：当电压低于一定水平，机器发出“请换电池”音提示，发射有效。

c.电压过低：当电压低于一定水平，机器发出“请换电池”音提示，按PTT键或声控发射都无效并发出“嘀”音提示。

a.Over-high voltage: Press [PTT] the key is invalid and the prompt sound.

b.Battery low alert: When the battery power is low, an alert tone will sound.

c.Below voltage: When the battery power is very low, Press [PTT] the key is invalid and the prompt sound.

Chinese / English Prompt

This radio includes Chinese and English two kinds of voice signal mode (channel knob must be switched to channel CH15), firstly turn off the radio, and then hold the PTT button and MONI button while turning on the power to convert another way. Each time you switch a way so repeated.

QT/DQT

The radio’s QT/DQT for each channel can be program by software. When the channel set the QT/DQT, the radio squelch open only if it receives a signal that uses the same QT or DQT. The radio squelch does not turn on if the same channel uses different QT / DQT calls, only lights in green, sub-CTCSS 50 groups, and digital sub-audio DCS 105 groups.

- QT：(50 Group)

67.0，69.3，71.9，74.4，77.0，79.7，82.5，85.4，88.5，91.5，94.8，97.4，100.0，

103.5，107.2，110.9，114.8，118.8，123.0，127.3，131.8，136.5，141.3，146.2，

151.4，156.7， 159.8，162.2，165.5，167.9，171.3，173.8，177.3，179.9，183.5，186.2，189.9，192.8，196.6，199.5，203.5，206.5，210.7，218.1，225.7，229.1，233.6，241.8，250.3，254.1。

- DQT:(210 Group)

D023N，D025N，D026N，D031N，D032N，D036N，D043N，D047N，D051N，D053N，D054N，D065N，D071N，D072N，D073N，D074N，D114N，D115N，D116N，D122N，D125N，D131N，D132N，D134N，D143N，D145N，D152N，D155N，D156N，D162N，D165N，D172N，D174N，D205N，D212N，D223N，D225N，D226N，D243N，D244N，D245N，D246N，D251N，D252N，D255N，D261N，D263N，D265N，D266N，D271N，D274N，D306N，D311N，D315N，D325N，D331N，D322N，D343N，D346N，D351N，D356N，D364N，D365N，D371N，D411N，D412N，D413N，D423N，D431N，D432N，D445N，D446N，D452N，D454N，D455N，D462N，D464N，D465N，D466N，D503N，D506N，D516N，D523N，D526N，D532N，D546N，D565N，D606N，D612N，D624N，D627N，D631N，D632N，D645N，D654N，D662N，D664N，D703N，D712N，D723N，D731N，D732N，D734N，D743N，D754N

D023I，D025I，D026I，D031I，D032I，D036I，D043I，D047I，D051I，D053I，D054I，D065I，D071I，D072I，D073I，D074I，D114I，D115I，D116I，D122I，D125I，D131I，D132I，D134I，D143I，D145I，D152I，D155I，D156I，D162I，D165I，D172I，D174I，D205I，D212I，D223I，D225I，D226I，D243I，D244I，D245I，D246I，D251I，D252I，D255I，D261I，D263I，D265I，D266I，D271I，D274I，D306I，D311I，D315I，D325I，D331I，D322I，D343I，D346I，D351I，D356I，D364I，D365I，D371I，D411I，D412I，D413I，D423I，D431I，D432I，D445I，D446I，D452I，D454I，D455I，D462I，D464I，D465I，D466I，D503I，D506I，D516I，D523I，D526I，D532I，D546I，D565I，D606I，D612I，D624I，D627I，D631I，D632I，D645I，D654I，D662I，D664I，D703I，D712I，D723I，D731I，D732I，D734I，D743I，D754I

三、Technical Specification

General

Frequency range                             UHF400-470MHz

Memory channels                            16 Groups

Operation voltage                          DC3.7V

Frequency stability                        ±2.5ppm

Operation temperature                      -20℃—+50℃

Mode of operation                          Simplex

Antenna impedance                           50Ω

Transmitter

RF Power                                    ＜5W

Maximum deviation(W/N)                      ≤5KHz/≤2.5KHz

Spurious emission                           ≤7.5uW

Adjacent channel power                      ≤-65dB/≤-60dB

SNR(W/N)                                    ≥-45dB/≥-40dB

QT/DQT (W/N)                                0.7±0.1KHz/0.4±0.1KHz

Modulation sensitivity                      8—12mV

Transmission current                        ≤1.2A

Receiver

Sensitivity                               -122dBm (12dB SINAD)

Audio power                               1W

Audio distortion                          ≤10%

Intermediation(W/N)                      ≥65dB/≥60dB

Adjacent channel selectivity(W/N)          ≥65dB/≥60dB

Clutter suppression                       ≥65dB

Receiver current                          ≤380mA

Note：The above specifications/technical instructions are for reference only because of the continuous improvement of the technology. Please refer to the physical object.