T7功能操作说明

一、功能简介：  Function Introduction：

1．工作频段：400 ~ 470MHZ    Frequency range：400-470MHZ

      - 16个存储信道         Up to 16 Channels

      - 中/英文语言报号      Chinese/English Language

4．50 组标准 CTCSS，105 组标准 DCS      50 Groups CTCSS，105 Groups of standard DCS

5．发射超时限制       Time-out timer(TOT)

# 6．宽/窄带功能       Wide/Narrow band

7．省电模式         Battery saving function

8．繁忙禁发功能     Busy Channel Lockout (BCL)

9．低电压告警      Low Battery Voltage Warning

10．电脑编程       Programmable by PC

二、功能操作  Function Operation

静噪级别   SQUELCH LEVEL

The purpose of Squelch is to mute the speaker when no signals are present (Squelch OFF). With the squelch level correctly set, you will hear sound only while actually receiving signals (Squelch ON). The squelch level can be adjusted via program software.

The frequency of squelch is 0 (on) -9 (deep), steps: 1.

省电功能   Battery Saving

a.ON: Entering into the battery saving state automatically without any operation for 10 seconds.

b.OFF: Battery saving is off.

繁忙锁定   Busy Channel Lockout (BCL)

Switch on or off each effective channel busy lockout function via PC programming software. When open this setting, signal from the current channel is banned to transmit.

语音播报  Voice broadcast

Turn on or off through PC software Settings.

ON: when the voice switch is on, the function and channel number are voice broadcast for each channel.

Off: when the voice switch is off, There will be no voice broadcast on each channel.

电池不足警告  Low Battery Voltage Warning

When the voltage is lower than the present value, a prompt "Please change battery" is issued every 10 seconds, press the PTT buttons is invalid and make "Di" sounds.

中英文选择

Set Chinese or English language through PC software.

声控发射  VOX

- a. VOX function is effective when the function is turn on.

- b. VOX gain level: When the sound level is higher than the set level of VOX, VOX automatically transmit. The VOX gain level can adjust from 1 to 5 levels.

- c. When a signal is received, VOX will not transmit even if the sound level is higher than the set VOX gain level.

亚音频QT/数字亚音频DQT    QT / DQT

The radio’s QT/DQT for each channel can be program by software. When the channel set the QT/DQT, the radio squelch open only if it receives a signal that uses the same QT or DQT. The radio squelch does not turn on if the same channel uses different QT / DQT calls, only lights in green, sub-CTCSS 50 groups, and digital sub-audio DCS 105 groups.

- 亚音频QT：（共50组）  QT：(50 in total)

67.0，69.3，71.9，74.4，77.0，79.7，82.5，85.4，88.5，91.5，94.8，97.4，100.0，

103.5，107.2，110.9，114.8，118.8，123.0，127.3，131.8，136.5，141.3，146.2，

151.4，156.7， 159.8，162.2，165.5，167.9，171.3，173.8，177.3，179.9，183.5，186.2，189.9，192.8，196.6，199.5，203.5，206.5，210.7，218.1，225.7，229.1，233.6，241.8，250.3，254.1。

- 数字亚音频DQT：（共 210 组） DQT:(210 in total)

D023N，D025N，D026N，D031N，D032N，D036N，D043N，D047N，D051N，D053N，D054N，D065N，D071N，D072N，D073N，D074N，D114N，D115N，D116N，D122N，D125N，D131N，D132N，D134N，D143N，D145N，D152N，D155N，D156N，D162N，D165N，D172N，D174N，D205N，D212N，D223N，D225N，D226N，D243N，D244N，D245N，D246N，D251N，D252N，D255N，D261N，D263N，D265N，D266N，D271N，D274N，D306N，D311N，D315N，D325N，D331N，D322N，D343N，D346N，D351N，D356N，D364N，D365N，D371N，D411N，D412N，D413N，D423N，D431N，D432N，D445N，D446N，D452N，D454N，D455N，D462N，D464N，D465N，D466N，D503N，D506N，D516N，D523N，D526N，D532N，D546N，D565N，D606N，D612N，D624N，D627N，D631N，D632N，D645N，D654N，D662N，D664N，D703N，D712N，D723N，D731N，D732N，D734N，D743N，D754N

D023I，D025I，D026I，D031I，D032I，D036I，D043I，D047I，D051I，D053I，D054I，D065I，D071I，D072I，D073I，D074I，D114I，D115I，D116I，D122I，D125I，D131I，D132I，D134I，D143I，D145I，D152I，D155I，D156I，D162I，D165I，D172I，D174I，D205I，D212I，D223I，D225I，D226I，D243I，D244I，D245I，D246I，D251I，D252I，D255I，D261I，D263I，D265I，D266I，D271I，D274I，D306I，D311I，D315I，D325I，D331I，D322I，D343I，D346I，D351I，D356I，D364I，D365I，D371I，D411I，D412I，D413I，D423I，D431I，D432I，D445I，D446I，D452I，D454I，D455I，D462I，D464I，D465I，D466I，D503I，D506I，D516I，D523I，D526I，D532I，D546I，D565I，D606I，D612I，D624I，D627I，D631I，D632I，D645I，D654I，D662I，D664I，D703I，D712I，D723I，D731I，D732I，D734I，D743I，D754I

技术指标  Technical Specification

General：

Frequency range                            UHF400-470MHz

Memory channels                            16 Groups

Operation voltage                          DC 3.7V

Frequency stability                        ±2.5ppm

Operation temperature                      -20℃—+50℃

Mode of operation                          Simplex

Antenna impedance                           50Ω

Transmitter:

RF Power                                    2W

Maximum deviation                          ≤±5KHz

Spurious emission                           ≤-60dB

Transmission current                        ≤1000mA

Preemphasis Character              6dB per octave

Receiver:

Sensitivity                                <0.16uv (12dB SINAD)

Squelch Sensitivity                         <0.2uv

Audio power                              ≥380mW

Audio distortion                           <10%

Intermediation(W/N)                      ≥65dB

Receiver current                          ≤380mA

Note：The above specifications/technical instructions are for reference only because of the continuous improvement of the technology. Please refer to the physical object.