T8设计方案

- 功能描述：Function Introduction:

- 直观的液晶LCD菜单式操作 LCD display monitor

- 电池：3.7V 1000mAh可充电锂电池 3.7V rechargeable lithium battery

- 频率范围：400~470MHz  Frequency range：400-470MHZ

- 宽/窄带选择：25KHz/12.5KHz    Wide/narrow band selection:25KHz/12.5KHz

- 射频输出功率：H:2W; L: 05W       Output power：H:2W   L: 05W

- 模拟亚音频CTCSS（50个）数字亚音频CDCSS（104组） 50 Groups CTCSS，104 Groups DCS

- 99个存储信道  Up to 99 Channels

- 发射限时 Time-out timer(TOT)

- 语音激活VOX免持通话  VOX

- 音频呼叫1750HZ、1450HZ、2100HZ。

- 背光灯开/关/Screen backlight

- 节电模式 Battery saving function

- 监听功能 Monitoring function

- 低电压指示/报警  Low Battery Voltage Warning/Indication

- 键盘提示音 Keyboard Beep

- 键盘锁  Keyboard Locks

- FM收音机87~108MHZ  FM Radio:87-108MHz

- LED照明灯  LED Flashlight

- 双守候功能 Dual standby function

- USB充电功能  Support USB charging function

- 电脑编程功能（专用数据线） Programmable by PC(Dedicated data line)

- 恢复出厂重置功能  Restore factory settings

- 多种工作模式可选  Multiple working modes are available

- 天气预报功能 Weather forecast function

- 10种呼叫音可选，体现个性化设计风格  10 ringtone are available

- 安全信息 Safety Information

- Please read the following information in order to use this two-way radio safely and efficiently.

- 1.The maintenance work of the two-way radio can only be carried out by professional technical personnel; assembly/dis assembly without permission is prohibited;

- 2.To avoid problems caused by electromagnetic interference and/or electromagnetic compatibility, please turn off the two-way radio in places with the sign “Please turn off two-way radio”, such as hospitals and other health care facilities.

- 3.When taking airplane, please turn off the two-way radio when the crew request.

- 4.In automobile with airbags, do not place the two-way radio in the inflation area of the airbags.

- 5.Turn off the two-way radio before entering the flammable and explosive environment;

- 6.Do not replace or charge the battery in flammable and explosive environments;

- 7.Turn off the two-way radio before approaching the blasting area and detonator area;

- 8.Do not use the two-way radio if the antenna is damaged, or else it may cause minor skin burns;

- 9.Do not expose the two-way radio to direct sunlight or near the heating device.

- 10.While the portable two-way radio is transmitting, keep the radio in a vertical position and keep the microphone about 5cm from the mouth.

- 11.Keep the two-way radio at least 2.5cm from the head or body during transmitting.

- 12.If you are wearing the portable two-way radio on your body, make sure that the antenna is at least 2.5cm away from the body when the two-way radio is transmitting.

- 13.If the two-way radio has any odor or smoke, turn off the power immediately and contact your dealer.

- 熟悉本机 Familiar with the machine

- 开关/音量调节旋钮 Power and Volume Knob

- 耳机插座  Earphone Socket

- 屏幕 LCD Screens

- 键盘 Keyboard

- 麦克风 Microphone

- 扬声器 Speaker

- 天线 Antenna

- PTT键 PTT Button

- USB和编程接口 Micro USB and Programmatic Interface

- LAMP按键   LAMP Button

- 手电灯  LED Flashlight

- 按键介绍 Description of the Buttons

- [Power and Volume]：	Power switch, adjust the volume.

- [PTT]：When transmitting, press this button and speak to the microphone; release the button to receive.

- [FM]：Press the button once to turn on the FM radio function, and then press the button again to turn off the FM radio function.

- [LAMP]：Press to turn on the flashlight, press it again to make the flashlight flash, and press it again to turn off the flashlight.

- [MENU]：Short press to enter menu setting function.

- 【MONI】：Monitoring Button,Short press for Monitoring function in normal mode ;

- In menu mode press this key to exit.

- [/Ringtone]：Long press this button to turn on/off keypad lock,Short press for ringtone call.(Note:After the keyboard lock is opened, the function key and confirm key operations are invalid)

- [△]：Short press this button to up;Long press to switch A/B frequency.

- [▽]：Short press this button to down;Long press to switch A/B frequency.

- 组合键操作功能  Combination key function

- Call Tone: Press and hold 【PTT】 button and【MONI】button to transmit 1750Hz tone signaling,Release the 【MONI】button to exit;Press and hold 【PTT】 button and【FM】button to transmit 1450Hz tone signaling;Press and hold 【PTT】 button and【】button to transmit 1750Hz tone signaling.

三．液晶图标及功能说明  LCD icon and function description

| Icon | Icon name | Description |  |
| --- | --- | --- | --- |
|  | Receiving and Transmitting | Receiving display:,Transmitting display: |  |
|  | Normal mode display frequency; PMR and FRS mode display channels | Display current channel, Press the[△]/[▽]button to select |  |
|  | Battery level indicator | The more cells in the display, the more power the battery |  |
|  | Channel number indication | 01-99 |  |
|  | Frequency offset direction | When "+" is displayed, it represents positive difference frequency; when "-" is displayed, it represents negative difference frequency.Do not display for the same frequency transceiver. |  |
| VOX | VOX indication | VOX has been turned on |  |
|  | Keypad lock indication | Keypad lock has been turned on(PTT button are not restricted） |  |
|  | Field Strength | The signal intensity is displayed when receiving, the first five grids are displayed when low power is transmitted, and the full grid is displayed when high power is transmitted. |  |
|  | Ringtone | 1-10 Category |  |
|  | Dual Standby |  |  |
|  | Low Power(Normal Mode) | When "L" is displayed, it represents low power transmission of the current channel; when is not displayed, it represents high power transmission. |  |
|  | Narrow Band (Normal Mode) | When "N" is displayed, it represents in the Narrow-band mode; when is not displayed, it represents in the broad-band mode |  |
|  | CTCSS | CTCSS:50 Group(Normal Mode) 38 Group(FRS/PMR) |  |
|  | CDCSS | CDCSS:104 Group(Normal Mode) 83 Group(FRS/PMR) |  |

四.功能简介 Brief of the Function

1、Output Power:

Normal Mode: Low 0.5W and High 2W;

FRS/PMR: 0.5W

2.Frequency Range:409(China) 467(TaiWan,China) 446(Europe) 442(USA) 477(Southeast Asia) 325(Philippines) 476-477(Australia) 433(Russia) 444(Norway) 448-447(Korea) 464(South Africa)

3、Channel:Up to 99 Channels(Preset according to different countries or regions)

五. 菜单内容及操作说明  Menu Item List

| Menu | Display Character | Set Options |
| --- | --- | --- |
| 1 | RXCODE（Receive CTCSS/DCS） | OFF，ctcss,dcs |
| 2 | TXCODE（Transmit CTCSS/DCS） | OFF，ctcss,dcs |
| 3 | SQL （Squelch level） | 0--9 （Factory set to level 3） |
| 4 | LIGHT （Backlight） | OFF/ON /Key |
| 5 | BEEP（Keyboard Beep） | OFF/ON |
| 6 | VOX | OFF/1---5 level |
| 7 | PWR（Transmission power） | HIGH/LOW |
| 8 | TDR（Dual standby） | ON/OFF |
| 9 | W/N(Wide band and narrow band) | WIDE/NARROW |
| 10 | SAVE(Power saving mode) | OFF/ON |
| 11 | TOT(Transmission time limit） | OFF/30/60/90/120/150/180S |
| 12 | BCLO(Busy channel lockout） | OFF/ON |
| 13 | RING(Ringtone) | OFF,10 ringtone are available |
| 14 | WEATHE(Weather forecast) | OFF/ON |
| 15 | VOICE(Language) | OFF/CHI(Chinese)/ENG(English) |

  - Select the RX CODE

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 01 and display RXCODE;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select the required code: OFF/67.0 - 754N;

Note:CTCSS:67Hz-254.1Hz(50 group) DCS:023N-754N/023I-754I(104 group)

- Press the[△]/[▽]button to select the CTCSS/DCS;

- Press [MENU] button to confirm;

6）Press[PTT]or[MONI]button exit to the menu mode.

  - Select the TX CODE

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 02 and display TXCODE;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select the required code: OFF/67.0 - 754N;

Note:CTCSS:67Hz-254.1Hz(50 group) DCS:023N-754N/023I-754I(104 group)

- 4）Press the[△]/[▽]button to select the CTC/DCS;

- 5）Press [MENU] button to confirm;

6）Press[PTT]or[MONI]button exit to the menu mode.

  - Squelch Level

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 03 and display SQL；

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:0/1/2/3/4/5/6/7/8/9.(The lowest level is 0,The highest level is 9 Usually set to level 3)

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - LIGHT背景灯长亮/键开启/关闭选择 Backlight

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 04 and display LIGHT;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:ON/OFF/KEY;

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - BEEP按键音设置开/关 Keyboard Beep

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 05 and display BEEP;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:ON/OFF;

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - VOX声控设置  VOX

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 06 and display VOX;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:OFF/1/2/3/4/5；

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

Note：The minimum sensitivity is 5,when background noise is high to use.

The highest sensitivity is 1,when background noise is low to use.

When VOX is opened,VOX icon appears in the upper left corner of the LCD.

  - POWER高/低功率选择  Transmitting Power

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 07 and display PWR;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:HIGH/LOW;

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - TDR  A/B双守功能   Dual Standby

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 08 and display TDR;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:OFF/ON;

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - W/N宽窄带选择 Wide Band and Narrow Band

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 09 and display WN;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:WIDE/NARROW；

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - SAVE省电选择 Power Saving Mode

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 10 and display SAVE;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:ON/OFF；

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - TOT发射限时选择 Transmission time limit(TOT)

- Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 11 and display TOT;

- 2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:OFF/30S/60S/90S/120S/150S/180S；

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

  - BCLO繁忙锁定选择 Busy Channel Lockout(BCLO)

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 12 and display BCLO;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:ON/OFF；

4）Press [MENU] button to confirm;

5）Press[PTT]or[MONI]button exit to the menu mode.

13.来电音（RING)    Ringtone

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 13 and display RING;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:OFF/1/2/3/4/5/6/7/8/9/10.

Note:Each time you select a call tone, you can listen to the call tone

- Press [MENU] button to confirm;

- 5）Press[PTT]or[MONI]button exit to the menu mode.Press the

button to send the selected ring tone.

14.天气预报 Weather Forecast

  - 天气预报菜单设置 Weather Forecast Menu Setting

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 14 and display WEATHE;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select:OFF/ON

4）Press [MENU] button to confirm;

5）Press[PTT]button exit to the menu mode.

-If the menu item is selected as ON, the radio enters the weather forecast mode.

-If the menu item is selected as OFF, the radio return to normal operation mode.

2.天气预报信道设置 Weather Forecast Channel Setting

In weather forecast receiving mode,Press the[△]/[▽]button to select weather forecast channel(CH 1 - CH 11).

Note:In weather forecast receiving mode, the radio receiving and transmitting function is deactivated.

3.天气预报频点 Weather Forecast Frequency

CH1: 162.550

CH2: 162.400

CH3: 162.475

CH4: 162.425

CH5: 162.450

CH6: 162.500

CH7: 162.525

CH8: 161.650

CH9: 161.775

CH10:161.750

CH11:162.000

15.语音提示 Language

1）Press the [MENU] button to enter into the MENU mode.press the[△]/[▽]button to select 15 and display VOICE;

2）Press the [MENU] button to enter the MENU item;

3）Press the[△]/[▽]button to select: OFF/CHI/ENG

4）Press [MENU] button to confirm;

5）Press[PTT]button exit to the menu mode.

-If the menu item is selected as OFF,the radio turn off the voice prompt.

-If the menu item is selected as CHI,the radio voice prompt is Chinese.

-If the menu item is selected as ENG,the radio voice prompt is English.

Note:In Normal Mode,the LCD display frequency and channel;

In PMR/FRS Mode, the LCD display channel number.

PMR模式模拟亚音码列表（HZ）  CTCSS List for PMR Mode

| Serial number | 音频（HZ） | Serial number | 音频（HZ） | Serial number | 音频（HZ） |
| --- | --- | --- | --- | --- | --- |
| 0 | OFF | 13 | 103.5 | 26 | 162.2 |
| 1 | 67.0 | 14 | 107.2 | 27 | 167.9 |
| 2 | 71.9 | 15 | 110.9 | 28 | 173.8 |
| 3 | 74.4 | 16 | 114.8 | 29 | 179.9 |
| 4 | 77.0 | 17 | 118.8 | 30 | 186.2 |
| 5 | 79.7 | 18 | 123.0 | 31 | 192.8 |
| 6 | 82.5 | 19 | 127.3 | 32 | 203.5 |
| 7 | 85.4 | 20 | 131.8 | 33 | 210.7 |
| 8 | 88.5 | 21 | 136.5 | 34 | 218.1 |
| 9 | 91.5 | 22 | 141.3 | 35 | 225.7 |
| 10 | 94.8 | 23 | 146.2 | 36 | 233.6 |
| 11 | 97.4 | 24 | 151.4 | 37 | 241.8 |
| 12 | 100.0 | 25 | 156.7 | 38 | 250.3 |

PMR模式数字亚音码列表 CDCSS List for PMR Mode

| Serial number | Code | Serial number | Code | Serial number | Code | Serial number | Code | Serial number | Code | Serial number | Code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 023 | 53 | 114 | 67 | 174 | 81 | 315 | 95 | 445 | 109 | 631 |
| 40 | 025 | 54 | 115 | 68 | 205 | 82 | 331 | 96 | 464 | 110 | 632 |
| 41 | 026 | 55 | 116 | 69 | 223 | 83 | 343 | 97 | 465 | 111 | 654 |
| 42 | 031 | 56 | 125 | 70 | 226 | 84 | 346 | 98 | 466 | 112 | 662 |
| 43 | 032 | 57 | 131 | 71 | 243 | 85 | 351 | 99 | 503 | 113 | 664 |
| 44 | 043 | 58 | 132 | 72 | 244 | 86 | 364 | 100 | 506 | 114 | 703 |
| 45 | 047 | 59 | 134 | 73 | 245 | 87 | 365 | 101 | 516 | 115 | 712 |
| 46 | 051 | 60 | 143 | 74 | 251 | 88 | 371 | 102 | 532 | 116 | 723 |
| 47 | 054 | 61 | 152 | 75 | 261 | 89 | 411 | 103 | 546 | 117 | 731 |
| 48 | 065 | 62 | 155 | 76 | 263 | 90 | 412 | 104 | 565 | 118 | 732 |
| 49 | 071 | 63 | 156 | 77 | 265 | 91 | 413 | 105 | 606 | 119 | 734 |
| 50 | 072 | 64 | 162 | 78 | 271 | 92 | 423 | 106 | 612 | 120 | 743 |
| 51 | 073 | 65 | 165 | 79 | 306 | 93 | 431 | 107 | 624 | 121 | 754 |
| 52 | 074 | 66 | 172 | 80 | 311 | 94 | 432 | 108 | 627 |  |  |

七．（PC）写频软件各国免执频点选择  Various country frequency to PC Programmable software

Note: In Normal Mode, You can write frequency according to requirements.(Frequency should comply with the laws and regulations of the country)

.