V1 V2 Function Operation Instruction

一、Function Introduction：

◆Frequency

FM：87–108 MHZ （RX）

UHF：400–470 MHZ （RX/TX）

◆ Up to 16 Channels

◆ Commercial FM radio

◆ Chinese/English Language

◆ 50 Groups CTCSS，105 Groups of standard DCS

◆ Time-out timer(TOT)

◆ Battery saving function

◆ Alarm function

◆ USB rapid charge

◆ Programmable by PC

Item Description

Note: The radio outlook based on the real radio.

A.LED Indicator: Green LED flashes when receive the signal, Red LED flashes when transmission.

B. Channel knob: Rotate the knob to choose the channel (CH1-CH15), CH16 is scan channel.

C.ON/OFF knob/Volume knob: Rotate the knob clockwise to turn on the radio, anticlockwise to turn off the radio.

By rotating to control the volume.

D.PTT switch: Press PTT and speak to the microphone to call, listen when release.

E. Monitor: When press Monitor key, the squelch opened and the background noise comes out; the squelch close when release the monitor key.

F.USB Rapid charge plug.

G. Speaker/Microphone rubber plug: Please cover when not use the speaker and microphone.

H. Insert loudspeaker.

J. Microphone

Battery Charging

1.Take out the charger, plug the AC（100-240V）50/60Hz, then Green LED flashes;

2. Plug the battery or the radio with battery on the charger; make sure the battery is connected with the charger.

3. The charging process initiates when the red LED lights, the charging time based on the battery capacity.

The green LED lights indicating the battery is fully charged.

4. Use the USB cable to plug to the radio, red LED lights when charging, green LED lights when fully charged.

二、Function Operation

SWITCHING POWER ON/OFF
Turn the power switches volume control clockwise. You will hear a beep and speech, indicating the transceiver is ON.
ADJUSTING THE VOLUME
Hold Monitor button down to listen to audio level while rotating the Power switch/Volume control. Rotate clockwise to increase and counterclockwise to decrease volume.
SELECTING THE CHANNEL
Turn the channel switch to select your desired channel. NO.16 is Scanning Channel
PTT TRANSMIT
To transmit, press and hold [PTT] and speak into the microphone in your normal tone of voice.
SQUELCH LEVEL
the purpose of Squelch is to mute the speaker when no signals are present (Squelch OFF). With the squelch level correctly set, you will hear sound only while actually receiving signals (Squelch ON). The squelch level can be adjusted via program software.

The frequency of squelch is 0 (on) -9 (deep), steps: 1.

SCANNING SWITCH

When walkie talkie is set as scan via software, turn the channel selector to channel 16, the transceiver will automatically detect the activity of scanning channel from CH1 to CH15 channel (each channel can bet set scan or not via software) , When the signal is scanned channel, the transceiver will automatically stop on the channel for the call.

- Walkie-talkie will stay on the signal channel, when the signal disappears and then after 15s, continue scanning the next channel.

- If there are less than 2 channels, it cannot scan.

- On channel 16, press and hold the PTT and MONI key, it will turn on the SCAN or turn off the SCAN.

- If the FM radio function is activated, it cannot scan.

VOICE PROMPT

a. Voice Prompt is effective when the function is turn on.

b. VOX gain level: When the sound level is higher than the set level of sound control gain, VOX automatically transmit. The gain level can adjust from 1 to 9 levels.

c. When a signal is received, VOX will not transmit even if the sound level is higher than the set gain level.

d. On channels CH1 ~ CH5, press and hold the PTT and MONI, then switch on the transceiver, thus to activate the VOX function ON/OFF.

MONITOR

When you are receiving and no signals present, the squelch function can mute the speaker, so you cannot hear the background noise .If you want to switch the squelch function OFF, press and hold (MONI) button .It is very helpful when you want to adjust the volume level and receive the week signals.

Battery Saving 
a.ON: Entering into the battery saving state automatically without any operation for 10 seconds,
b.OFF: Battery saving is off.

Busy Channel Lockout (BCL)
Switch on or off each effective channel busy lockout function via PC programming software. Whenopen this setting, signal from the current channel is banned to transmit.

Beep
a.  The beep is on, and the "Di" sounds is emitted for every rotation of the channel
b. The beep is off, and no "Di" sounds for every rotation of the channel

FM Radio Function

Press MONI and turn on radio, entering into FM radio mode. After FM radio starts, press MONI to search for frequency range (87-108MHZ).

In the working state of FM radio, if the current channel receives the carrier signal, press PTT button and turn off FM radio function.

Low Battery Voltage Warning
When the voltage is lower than the preset value, a prompt "Please change battery" is issued every 5 seconds, press the PTT button is invalid and make "Di" sounds.

Chinese / English Prompt

This radio includes male Chinese and English two kinds of voice signal mode (channel knob must be switched to channel CH15), firstly turn off the radio, and then hold the PTT button and MONI button while turning on the power to convert another way. Each time you switch a way, so repeated.

Time-out timer

The function of the time-out timer is to prevent the speaker from using a channel too long and continuously, and the interphone emits continuously too long and the body heats up. If you continuously transmit the interphone, the TOT limit time set by the machine is exceeded; the transceiver will stop transmitting (the time can be programmed by computer while the factory setting time is 1 minute). Release the PTT button and press PTT again to resume transmitting.

QT / DQT

The radio’s QT/DQT for each channel can be program by software. When the channel set the QT/DQT, the radio squelch open only if it receives a signal that uses the same QT or DQT. The radio squelch does not turn on if the same channel uses different QT / DQT calls, only lights in green, sub-CTCSS 50 groups, and digital sub-audio DCS 105 groups.

Note: Although using a channel with CTCSS / DCS sub-audio may be preferable to receiving unwanted calls, it does not mean your call is a secret.

三、Technical Specification

General：

Frequency range                            UHF400-470MHz

Memory channels                            16 Groups

Operation voltage                          DC3.7V

Frequency stability                        ±2.5ppm

Operation temperature                      -20℃—+50℃

Mode of operation                          Simplex

Antenna impedance                           50Ω

Transmitter:

RF Power                                    ＜5W

Maximum deviation(W/N)                      ≤5KHz/≤2.5KHz

Spurious emission                           ≤7.5uW

Adjacent channel power                      ≤-65dB/≤-60dB

SNR(W/N)                                    ≥-45dB/≥-40dB

QT/DQT(W/N)                                 0.7±0.1KHz/0.4±0.1KHz

Modulation sensitivity                      8—12mV

Transmission current                        ≤1.2A

Receiver:

Sensitivity                               -122dBm (12dB SINAD)

Audio power                               0.4W（8R Load）

Audio distortion                          ≤10%

Intermodulation(W/N)                      ≥65dB/≥60dB

Adjacent channel selectivity              ≥65dB/≥60dB

Clutter suppression                       ≥65dB

Receiver current                          ≤380mA